Water Quality Test Results from  Meghna River at  Bisnandi ( 4 dates ;Feb 15, 22  March 01 & 08 , 2014,samples collected by the existing DWASA water monitoring contractor  and at Baidder Bazar intake , Haria, Sonargaon, Narayanganj  (  3 dates : July 13, August 24  & Sept  28 ,2013),water samples collected by Egis Eau-IWM & water Quality Test Report from Feasibility Study volume 1 Final Report August 2011

| SL | Parameter | Unit | Bangladesh Drinking water Standard ECR ‘92 | Meghna River at Bisnandi |  |  |  | Meghna River at Baidder Bazar |  |  | Meghna ,Bisnandi March 22-Aug11,2010 |  | Meghna Baidarbaza March22- Aug11,2010 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Feb’15, 2014 | Feb’22, 2014 | March’01, 2014 | March’08, 2014 | July’13, 2013 | August’24, 2013 | September’28, 2013 | Max * | Min * | Max * | Min * |  |
| 1 | Turbidity | NTU | 10 | 1.26 | 1.88 | 0.09 | 2.45 | 6.87 | 17.1 | 13.00 | 52.1 | 3.95 | 45.5 | 3.75 |  |
| 2 | TDS | mg/l | 1000 | 48 | 51 | 55 | 62 | 35 | 20 | 27 | 236 | 50 | 110 | 37 |  |
| 3 | TSS | mg/l | 10 | 1.00 | 4.6 | 2.00 |  | 11 | 13 | 24 | 9 | 7 | 18 | 10 |  |
| 4 | COD | mg/l | 4 |  |  |  |  | 8 | 7 | 8.5 | 6 | 6 | 21 | 7 |  |
| 5 | BOD | mg/l | 0.2 |  |  |  |  | 1.0 | 0.4 | 0.6 |  |  |  |  |  |
| 6 | Iron | mg/l | 0.3-1.0 |  |  |  |  | 0.38 | 0.44 | 0.32 |  |  |  |  |  |
| 7 | Ammonia | mg/l | - | 0.5 | 0 | 0 | 0 | 0.001 | 0.004 | 0.001 | 0.56 | 0.2 | 0.55 | 0,2 |  |
| 8 | Ammonium | mg/l | 0.5 |  |  |  |  | 0.23 | 0.354 | 0.274 |  |  |  |  |  |
| 9 | Aluminum | mg/l |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Colour | Pt-Co | 15 | 5.8 | 4.2 | 3.2 |  | 13 | 19 | 15 |  |  |  |  |  |
| 11 | Hardness | mg/l | 200-500 | 72 | 60 | 52.5 | 52.5 | 16 | 20 | 36 |  |  |  |  |  |
| 12 | Alkalinity | mg/l |  | 75 | 60 | 60 | 75 | 21 | 25 | 30 |  |  |  |  |  |

Note*  Source FS volume 1 Final report August 2011